PICO-2W PDX

Digital Modes Transceiver

Step-1 User & Operations Manual

Station: VU2UPX · Grid: MK69KE · Pico 2W (RP2350)

Firmware: PICO2W_PDX_Step1 · Manual version 1.4.0 · 2026-09-22

# 1. Purpose of Step-1

Step-1 is the first milestone of the PICO-2W PDX project: a standalone digital-modes beacon platform on the Raspberry Pi Pico 2W with Si5351, OLED, optional TFT TX log, WiFi/NTP, web UI (Beacon On/Off), and FT8/WSPR beacons.

## 1.1 Hardware

Photo of the assembled unit (placeholder until replaced).

Figure 1 — Hardware / PCB [placeholder]

# 2. Web interface

Open the STA IP shown on the OLED, or http://192.168.4.1 in AP/config portal mode. Screenshots below are rendered from the firmware web page markup.

## 2.1 Main page (Beacon OFF)

Figure 2 — Web UI (Beacon OFF)

## 2.2 Main page (Beacon ON)

Figure 3 — Web UI (Beacon ON)

## 2.3 Soft buttons

| Button | Action |
| --- | --- |
| Mode (A) | Cycle FT8 → FT4 → JS8 → WSPR |
| Band Up / Dn | Change HF band |
| Tune / PTT | Toggle carrier TX / RX |
| Menu / Cal | Enter menu / calibration |
| Beacon ON/OFF | Enable/disable beacon scheduler (default OFF) |

# 3. Displays

## 3.1 OLED (primary)

| Line | Content |
| --- | --- |
| 1 | Mode + IST time |
| 2 | Dial frequency |
| 3 | RX/TX, band, BCN/--- or TX progress |
| 4 | STA or AP IP |

Figure 4 — OLED display [placeholder]

## 3.2 TFT TX log

Format: MODE  BAND  DD-MM-YYYY  HH:MM:SS (IST). Header shows BEACON ON/OFF.

Figure 5 — TFT TX log [placeholder]

# 4. Front-panel controls

| Control | Short | Long |
| --- | --- | --- |
| Switch A | Mode cycle | Save EEPROM & exit menu/cal |
| Switch B | Band up | Cancel menu/cal |
| Switch C | Band down | Cancel menu/cal |
| Encoder SW | — | Enter CAL / menu |
| Encoder | Adjust corr in CAL | — |

# 5. FT8 and WSPR beacons

Only when Beacon is ON and mode is FT8 or WSPR. Never concurrent.

| Mode | Timing | Notes |
| --- | --- | --- |
| FT8 | Once per FT8_TX_EVERY_MIN at UTC ~:00 | 79 symbols, Costas |
| WSPR | Even UTC minutes; every WSPR_TX_EVERY_MIN | Type 1 call+grid+dBm |

# 6. Configuration (config.h)

| Define | Purpose |
| --- | --- |
| MY_CALLSIGN / MY_GRID4 | Station identity |
| WSPR_DBM / WSPR_TX_EVERY_MIN | Power and WSPR interval |
| FT8_TX_EVERY_MIN | FT8 interval |
| DISPLAY_OPTION | 1 = TFT log |
| WIFI_AP_SSID / PASS | Config portal |

# 7. Getting started

1. Install libraries; configure TFT_eSPI if needed.

2. Upload with Pico 2W + Pico SDK USB stack.

3. Connect WiFi; confirm IST on OLED.

4. Calibrate Si5351 (Section 8).

5. Select mode; Beacon ON; confirm TFT log.

# 8. Si5351 calibration procedure

CAL mode outputs 10.000 MHz on CLK1. RX_EN and Tx/Rx relay off; CLK0/CLK2 disabled.

Figure 6 — Calibration procedure flow

Figure 7 — Calibration bench setup [placeholder]

## 8.1 Step-by-step

1. Beacon OFF. Long-press encoder → CAL (OLED: 10MHz CLK1).

2. Measure CLK1; rotate encoder until counter reads 10,000,000 Hz.

3. Long-press Switch A to save and exit. B or C cancels.

4. Optional: set correction on web UI → Save EEPROM.

# 9. RF / antenna

Use a dummy load or legal antenna. Match WSPR_DBM to real power.

Figure 8 — Antenna / dummy load [placeholder]

# 10. Operating notes

Keep Beacon OFF while calibrating.

WSPR TX ~111 s — monitor PA temperature.

FT8 free-text may decode as free text in WSJT-X.

# 11. Known limits

No DC RX path yet

No USB Audio for WSJT-X

No standard FT8 CQ packing yet

No Si4732/PCF8574 automation

# 12. Manual versioning

This document is produced by the automated builder in manual_build/. Version is read from VERSION; figures from figures/manifest.json.

| Command | Effect |
| --- | --- |
| ./bump_version.sh patch | 1.4.0 → 1.4.1 and rebuild |
| ./bump_version.sh minor | 1.4.0 → 1.5.0 and rebuild |
| ./bump_version.sh major | 1.4.0 → 2.0.0 and rebuild |
| python3 build_manual.py | Rebuild current VERSION only |
| python3 build_manual.py --check | Verify all figures exist |

# 13. Document control

| Field | Value |
| --- | --- |
| Manual version | 1.4.0 |
| Build date | 2026-09-22 |
| Station | VU2UPX · MK69KE |
| Firmware | PICO2W_PDX_Step1 |
| Figures total | 8 |
| Screenshots | 2 |
| Placeholders | 5 |

— End of Step-1 User & Operations Manual v1.4.0 —